See Owner’s manual pp 89-92

Initial state 0x00, 0x40, 0x80.

Full id range (instrument backlite max, auto) - RX:

IDx00: FC 7F

IDx02: FF FC FD FF 3D FF FE FF

IDx06: 93 FC 0D FF 3D FF FE FF

IDx10: 50 FC FD FF 3D FF FE FF

| RX (19200 baud) |  |  |
| --- | --- | --- |
|  | Pressed: B0 B1 B2 B3 B4 B5 B6 B7 IDx02: FF FD FD FF 3D FF FE FF IDx06: CE FD 0D FF 3D FF FE FF IDx10: 50 FD FD FF 3D FF FE FF Released: IDx02: FF FC FD FF 3D FF FE FF IDx06: 93 FC 0D FF 3D FF FE FF IDx10: 50 FC FD FF 3D FF FE FF | Front Fog Lamp - FrntFogLamp B1.0 |
|  | Pressed IDx02: FF FE FD FF 3D FF FE FF IDx06: 29 FE 0D FF 3D FF FE FF IDx10: 50 FE FD FF 3D FF FE FF Released IDx02: FF FC FD FF 3D FF FE FF IDx06: 93 FC 0D FF 3D FF FE FF IDx10: 50 FC FD FF 3D FF FE FF | Rear Fog Lamp - ReFogLamp B1.1 |
|  | Initial pos <0>: IDx02: FF FC FE FF 3D FF FE FF IDx06: 59 FC 0E FF 3D FF FE FF IDx10: 50 FC FE FF 3D FF FE FF Switched to Auto: IDx02: FF FC FD FF 3D FF FE FF IDx06: 93 FC 0D FF 3D FF FE FF IDx10: 50 FC FD FF 3D FF FE FF | Automatic Headlight Control - AutHdLiCtrlMod ~B2.1 |
|  | See previous one | Lamps Off - LampOffMod ~B2.0 |
|  | Initial pos <0>: IDx02: FF FC FE FF 3D FF FE FF IDx06: 59 FC 0E FF 3D FF FE FF IDx10: 50 FC FE FF 3D FF FE FF Switched to Parking Lamps IDx02: FF 7C FF FF 3D FF FE FF IDx06: BE 7C 0F FF 3D FF FE FF IDx10: 50 7C FF FF 3D FF FE FF | Parking lamps - PrkgLiMod ~B1.7 |
|  | Initial pos <Parking Lamps>: IDx02: FF 7C FF FF 3D FF FE FF IDx06: BE 7C 0F FF 3D FF FE FF IDx10: 50 7C FF FF 3D FF FE FF Switched to Low Beam position: IDx02: FF BC FF FF 3D FF FE FF IDx06: C1 BC 0F FF 3D FF FE FF IDx10: 50 BC FF FF 3D FF FE FF | Low Beam - LoBeamMod ~B1.6 |
| Switch failure (after disconnection of the top panel) | IDx02: FF BC FF FF 7D FF FE FF IDx10: 50 BC FF FF 7D FF FE FF IDx06: 55 BC 0F FF 7D FF FE FF | Switch Failure B4.2 (0 – normal, 1 – failure) |
|  | Top position: IDx02: FF BC FF FF 7D FF FE FF IDx06: 55 BC 0F FF 7D FF FE FF IDx10: 50 BC FF FF 7D FF FE FF Bottom position: IDx02: FF BC FF FF 7D FF 00 FF IDx06: 58 BC 0F FF 7D FF 00 FF IDx10: 50 BC FF FF 7D FF 00 FF | Instrument Lighting B6: 0 = 0% ... 254 = 100% 255 = failure |
| Pot failure (short to ground) | IDx02: FF BC FF FF 7D FF FF FF IDx06: 55 BC 0F FF 7D FF FE FF IDx10: 50 BC FF FF 7D FF FE FF |  |
| TX (19200 baud) |  |  |
|  | IDx2B: 00 xx 00 00 | Backlite B1: 0 = 0% ... 254 = 100% 255 = off (no valid data) |

Summary

RX BR=19200, ID = 0x0A, length = 8, Enhanced Checksum – SWITCHES state - SwtSt

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BYTE 0 |  |  |  |  |  |  |  |
| BYTE 1 |  |  |  |  |  |  |  |
| PrkgLiMod 1 – off, 0 – on | LoBeamMod 1 – off, 0 – on |  |  |  |  | FrntFogLamp 0 – off 1 - on | ReFogLamp 0 – off 1 - on |
| BYTE 2 |  |  |  |  |  |  |  |
|  |  |  |  |  |  | AutHdLiCtrlMod 1 – off, 0 – on | LampOffMod 1 – off, 0 – on |
| BYTE 3 |  |  |  |  |  |  |  |
| BYTE 4 |  |  |  |  |  |  |  |
|  |  |  |  |  | LiSwtFailr 0 – norm., 1 - failure |  |  |
| BYTE 5 |  |  |  |  |  |  |  |
| BYTE 6 |  |  |  |  |  |  |  |
| DshbBri — Instrument Claster Brightness (0-254 = 0-100%, 255 – not valid data/off) |  |  |  |  |  |  |  |
| BYTE 7 |  |  |  |  |  |  |  |

TX BR=19200, ID = 0x2B, length = 4, Enhanced CS – BACKLITE control - BackliteCtrl

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BYTE 0 |  |  |  |  |  |  |  |
| BYTE 1 |  |  |  |  |  |  |  |
| BackliteBri — Backlite Brightness (0-254 = 0-100%, 255 – not valid data/off) |  |  |  |  |  |  |  |
| BYTE 2 |  |  |  |  |  |  |  |
| BYTE 3 |  |  |  |  |  |  |  |